Detail Tender for Electrical Work LAN Cabling work of Azamghad

Branch-Baroda U.P. Bank Ro-Azamghad

| Sl.No. | Particulars | Qty. | Unit | Rate/Unit Rs. P. | Amount Rs. P. |
| --- | --- | --- | --- | --- | --- |
| 1. | S/F of plug socket board consisting of 3 plug 5 Amp, three pin, three switch and one indicator with 1.5 sq mm separate circuit wiring for computer . | 11 | No |  |  |
| 2. | S/F of UPS earthing-S/burying of Dual pipe technology (Pipe in Pipe) earthingelectrode having GI 80mm O/D pipe with 25 micron coating, 3metres long. The space between two pipes to be filled with Highly conducive & Non corrosive chemical compound. The vacant space between pipe & hole shall also be filled with Compound all around. The job also includes S/F of GI strip clamp with two holes (with SS/GI bolts & nuts) on the electrode& taking two earthing leads of 4mm dia copper wire (Assumed length of 10m -beyond 10m to be paid extra)upto Main panel complete in all respects with guaranteed E/N voltage of less than 2 volts and resistance less tahn 1ohms. The 4mm dia Cu. earth wire shall be terminated on a separate 6way earthlink. The connections for Panel earthing& earth bus in the panle should be taken from this earthlink | 2 | No |  |  |
| 3 |  | S/W 10 Amp, Plug point with 2.5 sq mm copper wire in counter with continuous earthing wire complete in all respect with modular switches /socket. | 6 | No |  |  |
| 4 |  | S/F of LED Batten 20 watt light Fitting with 1.5 sq mm wiring complete in all respect including modular switches | 25 | No |  |  |
| 5 |  | S/F of 20 Amp plug point with 2.5 sq mm wiring including modular switches /socket with continuous circuit wiring. | 6 | No |  |  |
| 6 |  | Providing & Fixing of wall mounted fans (400dia) complete with 1.5 Sq mm wiring and modular switches. The work to include making hole in wall | 5 | No |  |  |
| 7 |  | Supplying and Fixing of cabin fans (400Dia) in cash cabin/ in 1.5 sq mm wiring and modular switches. | 1 | No |  |  |
| 8 |  | Supplying and Fixing of circuit wiring for computer in 2.5 sq mm copper wire. | 10 | No |  |  |
| 9 |  | Supplying and Fixing of wiring for lights and fans 1.5 sq mm copper wire complete in all respect. | 12 | No |  |  |

| Sl.No. | Items of Work | Qty. | Unit | Rate/Unit Rs. P. | Amount Rs. P. |
| --- | --- | --- | --- | --- | --- |
| 10 | Supplying and Fixing of exhaust fan in toilet/hall 15 1nch dia 1.5 sq mm wiring and modular switches. | 2 | No |  |  |
| 11 | S/F of ceiling fan (1200 dia) with hooks in rooms with adding rod of suitable length wherever necessary including 1.5 sq mm wiring and modular switches and regulator | 9 | No |  |  |
| 12 | S & F of 12 way SPN DB double door with 10No SP MCB and 1 no DPMCB 40 AMP for computer | 2 | No |  |  |
| 13 | Sub main wiring of UPS input and UPS output power supply with 2 no 6 sq mm No. 4 sq mm copper flexible wire in PVC conduit pipe. (payment as/actual length on site) | 45 | Mtr |  |  |

| Sl.No. | Items of Work | Qty. |  | Unit | Rate/Unit Rs. P. | Amount Rs. P. |
| --- | --- | --- | --- | --- | --- | --- |
| 14 | Supply and fixing on board 3 no KIT KAT cut out 200 amp with neutral link on board | 1 | Job |  |  |  |
| 15 | S/F 63 Amp Changeover | 1 | No |  |  |  |
| 16 | S/F 63 Amp Main Switch | 1 | No |  |  |  |
|  | LAN Cabling |  |  |  |  |  |
| 17 | Supplying and drawing CAT 6 PVC insulated and Sheathed LAN cable through PVC conduit including testing, commissioning etc.as required. | 240 | Mtr |  |  |  |
| 18 | Supply and fixing of RJ modem Jack for computer including making connection, soldering etc | 11 | No |  |  |  |
| 19 | Supply and Fixing of Patch Card with CAT 6 Jack | 11 | No |  |  |  |
|  |  |  |  |  | Total Rs. |  |

GRAND TOTAL -------------------

( Rs. -------- Lacs ------- Thousand Only.)

Total Work Furnishing Rs. --------------------- + Total Work Electrical Rs. -------------------------

Combined Total Work Furnishing  & Electrical Rs. ------------------------- (---------- Lacs -----------Thousand -------------- Only